# T-Map: Pre-Sale Product Roadmap

Industrial AI Safety Monitoring Platform — path from working prototype to sellable SaaS product

## 1. Where the product stands today

Built and working:

- YOLOv8 (TensorRT FP16) + ByteTrack detection and tracking

- Homography-based perspective transform → 2D blueprint + GPS coordinates

- 4-tier "Diamond Cascade" alert pipeline: rule engine → IVF visual-redundancy filter → Llava triage VLM → Qwen 2.5-VL heavy verification

- React + Leaflet live dashboard with clickable camera markers

- AES-256 encrypted camera credential storage

- SQLite/PostgreSQL dual-mode storage

Known gaps (from real testing):

- Cross-camera Re-ID (position-based, appearance-independent) doesn't work reliably yet — removed for now

- System tops out around 20 cameras continuously on current hardware/architecture; real industrial sites need 100s

- Camera calibration is manual (MS Paint pixel coordinates) — not scalable to onboard new sites quickly

- No multi-tenant SaaS layer, no compliance/audit export, no TTS intervention module yet

This roadmap sequences the work needed to close those gaps before your first paid pilot, and separates "must-fix before any sale" from "can follow after the first customer."

## 2. Guiding principle

Don't try to ship the 100-camera enterprise vision first. Land a realistic pilot (10–30 cameras, one facility zone) on the architecture you can actually support today, and use that revenue + real usage data to fund the scaling work. Promising more than the current architecture can do is the fastest way to lose a customer mid-pilot.

## 3. Phase 0 — Stabilize the core (2–4 weeks)

Goal: the current single-node system is reliable and benchmarked, so every later estimate and price is based on real numbers, not guesses.

- Load-test the current pipeline properly: fix FPS, ramp camera count, log GPU/VRAM usage and alert-queue depth until it backs up. Get real numbers for your GPU tiers (you already have a real data point: 4GB GPU / 16GB RAM → 8–10 cameras well, ~20 max).

- Batch YOLO inference across multiple camera streams in one forward pass instead of per-camera sequential loops — biggest single throughput win available on your current hardware.

- Make FPS adaptive: lower sampling (2–3fps) for low-risk zones, higher only where needed (near machinery, elevated work, restricted zones).

- Document the Re-ID failure mode precisely (what breaks, under what conditions) — needed both to fix it and to eventually support the patent disclosure.

- Ship v1 explicitly without Re-ID. Per-camera detection + verified alerts + blueprint map is already sellable on its own.

Exit criteria: you can say, with a straight face and real numbers, "our current setup reliably supports up to N cameras per GPU node."

## 4. Phase 1 — Make it installable (3–5 weeks)

Goal: a new customer site can be onboarded without you personally configuring everything by hand.

### 4.1 Easy calibration

- Replace manual pixel-coordinate calibration with a guided UI: customer (or your installer) clicks 4 reference points on the live camera frame, then the same 4 points on the blueprint image — homography matrix computed automatically.

- Add a calibration preview mode (overlay the projected grid on the live feed so misalignment is caught before going live).

- Store calibration profiles per camera in the DB, versioned, so a camera reposition doesn't require redoing the whole site.

### 4.2 Data & camera integration

- Standardize on ONVIF for camera discovery/connection where possible, not just raw RTSP URLs — most industrial CCTV vendors support it, and it cuts integration time per site.

- Define your company-DB integration contract clearly: what you read (employee shift data? zone maps?) vs. what you only need from the camera network. Keep the required integration surface as small as possible — every extra system you need write-access to is a longer security review for the customer.

- Build a camera health-check layer (stream up/down, calibration drift, credential expiry) — customers will ask "how do I know if a camera silently went offline."

Exit criteria: a new site (a handful of cameras) can go from signed contract to live dashboard in days, not weeks, without custom engineering per customer.

## 5. Phase 2 — SaaS & multi-tenancy (3–4 weeks)

Goal: the product can serve more than one customer without cross-contamination.

- Tenant isolation: separate data, camera credentials, and alert history per customer account (schema-per-tenant or row-level isolation, not shared global state).

- Auth & roles: at minimum — admin, safety manager, viewer-only. Enterprise buyers will ask about this in the first call.

- Usage metering per tenant (camera count, alert volume) — this becomes your billing basis.

- Provisioning flow: adding a new customer/site shouldn't require a manual deploy.

- Decide your deployment model now, since it shapes pricing and infra cost:

  - Centrally hosted SaaS (you run the GPUs, customer just points streams at you) — easier to sell and update, but GPU cost is on you.

  - On-prem GPU appliance (customer buys/leases a sized box) — shifts compute cost to the customer, harder to update remotely.

  - A hybrid (edge detection on-prem, VLM reasoning centralized) is likely your long-term answer — see Phase 4.

Exit criteria: you could onboard a second and third customer on the same infrastructure without manual reconfiguration each time.

## 6. Phase 3 — Security & compliance (2–3 weeks, can run parallel to Phase 2)

Goal: pass a mid-size industrial customer's security/legal review.

- Extend your existing AES-256 credential encryption to all sensitive data at rest, not just camera credentials.

- Encrypt data in transit (TLS everywhere — dashboard, WebSocket feeds, API).

- Write a data-retention policy (how long video clips/snapshots/telemetry are kept, and make it configurable per customer).

- Privacy-by-design for worker tracking — do this before you sell, not after a complaint:

  - Decide whether tracked identities are ever tied to named individuals or stay anonymized (global_id only).

  - Document consent/disclosure requirements — this varies by jurisdiction (EU works-council requirements are stricter than most US states) and is a real blocker in enterprise legal review if unaddressed.

  - Offer an aggregate-only reporting mode for customers who don't want individual tracking at all.

- Basic SOC2-readiness checklist (even before pursuing certification) — access logging, credential rotation policy, incident response plan. Enterprise buyers increasingly ask for this even from small vendors.

Exit criteria: you have a one-page security/privacy summary you can hand to a customer's IT/legal team without scrambling.

## 7. Phase 4 — Scale beyond one GPU box (ongoing, starts once Phase 0–3 pilots are live)

Goal: path from ~20 cameras to 100+ per site.

- Split detection (edge) from reasoning (central): run lightweight YOLO on cheap edge devices near cameras, send only flagged candidate crops to your central VLM cluster — cuts both compute and bandwidth dramatically.

- Horizontal scale-out: N cameras per GPU node (use your Phase 0 benchmark number), add nodes as camera count grows, with a router assigning cameras to nodes.

- Autoscaling for VLM workers (Celery/Redis queue you already have is a good base) so you're not paying for peak GPU capacity 24/7.

- Revisit Re-ID once detection scale is solid — don't let it block earlier phases.

Exit criteria: a documented, repeatable way to add GPU capacity as a customer's camera count grows, without redesigning the pipeline each time.

## 8. Phase 5 — Differentiation & IP (can start anytime, doesn't block sales)

- File a provisional patent around the position-based (appearance-independent) cross-camera Re-ID method once it's working reliably — this is your strongest, most defensible claim.

- Document the Diamond Cascade's compute-efficiency numbers (false-positive rate, GPU-cost-per-verified-alert vs. naive single-VLM approach) — useful both for a patent disclosure and as a sales/marketing number.

- Build the compliance-audit export (structured, timestamped alert reasoning formatted for OSHA/insurance review) — a sellable feature on its own, not just nice-to-have.

- TTS/voice intervention module — real differentiator against pure-alerting competitors, but not a blocker for first pilots.

## 9. Pricing & packaging (do this once Phase 0 gives you real numbers)

- Convert your camera-per-GPU benchmark into concrete tiers (e.g., "Starter: up to 10 cameras," "Standard: up to 30 cameras") rather than flat "unlimited camera" pricing — protects margin as customers scale.

- Price per-camera-per-month, roughly tracking your measured GPU cost per camera.

- Decide upfront what's included vs. add-on: compliance export, TTS intervention, and Re-ID (once fixed) are natural premium-tier features.

## 10. Suggested sequencing (rough, not a hard deadline)

| Weeks | Focus |
| --- | --- |
| 1–4 | Phase 0: benchmark, batch inference, ship v1 without Re-ID |
| 4–8 | Phase 1: calibration UI, camera integration standardization |
| 6–10 | Phase 2 (parallel): multi-tenancy, auth, provisioning |
| 6–10 | Phase 3 (parallel): security hardening, privacy policy |
| 10–12 | First paid pilot (10–30 cameras, one facility) |
| Ongoing after first pilot | Phase 4: edge/horizontal scaling, funded by pilot revenue and real usage data |
| Anytime, low-priority | Phase 5: patent filing, compliance export, TTS |

## 11. Bottom line

You don't need the 100-camera enterprise architecture to make your first sale. You need: a stable, benchmarked single-node system, an installable onboarding flow (calibration + integration), basic multi-tenancy, and a security/privacy story that survives a legal review. That combination is achievable in roughly 10–12 weeks and is enough to land a real pilot — the scaling work belongs after that, funded by what you learn from it.